YCT-33 Automatic Top and Bottom Labeling Machine:

Basic Use:

① This labeling machine is good for all kinds of specifications box,cover,battery,carton and irregular and flat base products labeling,such as food can, plastic cover,box,toy cover and plastic box the shaped like an egg.

②It can achieve top and bottom labeling, full coverage labeling,partial accurate labeling,vertical multi-label labeling and horizontal multi-label labeling, widely used in carton, electronic, food and packaging materials industries.

③Additional functions:

- configuration code printer or ink-jet printer, when labeling, print clear production batch number, production date, effective date and other information,coding and labeling will be carried out simultaneously.

- configuration printer,change printer contents at any time,realize the function of printing and labeling at the same time.

- Automatic feeding function (combined with product consideration);

- Automatic material collection function (combined with product consideration);

- Increase labeling device;

④The adjust method is simple: 1.Adjust the top labeling mechanism higher about 2mm than the product 2.Adjust the bottom labeling mechanism lower about 2mm than product bottom. 3.Adjust the conveyor belt and labeling speed on the touch screen so that they want to match. 4. Adjust the position of the sensor so that each label can be completely run out. 5.Adjust the height of the brush , let brush slight touch the labeling surface of the product.

⑤Floor space about 1.74 stere.

⑥Machine Support Customization.

The labeling machine has high labeling accuracy and good quality, Applicable to the requirements of high precision, high output products, and it is difficult to see the error with the naked eye.

Working principle:

1.Click star on the touch screen.

2.The product placed next to the guardrail, then the conveyor belt moves the products forward.

3.When the sensor detects that the products has reached target location,the machine will send out the label and the brush attaches the label to the product,a labeling process is completed.

Label specification:

①Applicable labels: sticker label, film, electronic supervision code, bar code.

②Applicable products: Products that are required to be labeled on flat, arc-shaped,round, concave, convex or other surfaces.

③Application industry: Widely used in cosmetics, food, toys, chemical, electronics, medicine and other industries.

④Application examples: shampoo flat bottle labeling, packaging box labeling, bottle cap, plastic shell labeling, etc.

label production requirements

1. The gap between the label and the label is 2-3mm;

2. The distance between the label and the edge of the bottom paper is 2mm;

3. The bottom paper of the label is made of glassine, which has good toughness and prevents it from breaking (to avoid cutting the bottom paper);

4. The inner diameter of the core is 76mm, and the outer diameter is less than 280mm, arranged in a single row.

The above label production needs to be combined with your product. For specific requirements, please refer to the results of communication with our engineers!

Technical parameters:

| Parameter | Date |
| --- | --- |
| Label Specification | Adhesive sticker,transparent or opaque |
| Labeling Tolerance | ±1mm |
| Capacity(pcs/min) | 40~120 |
| Suit bottle size(mm) | L:40~400 W:40~200 H:0.2~150;Can be customized |
| Suit label size(mm) | L:6-340mm;W(H):15-130 |
| Machine Size(L*W*H) | 1930*695*1390(mm) |
| Pack Size(L*W*H) | 1950*730*1450(mm) |
| Voltage | 220V/50(60)HZ;Can be customized |
| Power | 1030W |
| N.W(KG) | 180.0kg |
| G.W(KG) | 330.0kg |
| Label Roll | ID:Ø76mm; OD:≤240mm |

## Main Structures:

| No. | Structure | Function |
| --- | --- | --- |
| 1 | Conveyor | Transmit product |
| 2 | Top Labeling Head | Labeling on the top,core of the labeler,including label-winding and driving structure |
| 3 | Bottom labeling Head | Labeling on the bottom,core of the labeler,including label-winding and driving structure |
| 4 | Product Sensor | Detect product |
| 5 | Label-peeling Plate | Peel label from the release paper |
| 6 | Brush | Smooth labeled surface |
| 7 | Touch Screen | Operation and setting parameters |
| 8 | Strengthening Device | Press labeled product to strengthen labeling |
| 9 | Collection Plate | Collect the labeled products |
| 10 | Electric Box | Place electronic configurations |
| 11 | Double Side Guardrails | Keep products go straightly,can be adjusted accordingto product size. |

## Features:

1 ) Control System: Japan Panasonic control system , with high stability and extremely low failure rate.

2 ) Operation System: Touch screen , directly visual interface easy operation.Chinese and English available . Easily to adjust all electrical parameters and have counting function , which is helpful for production management.

3) Detection System: German LEUZE/Italian Datalogic label sensor and Japanese Panasonic product sensor , which are sensitive to label and product , thus ensure high accuracy and stable labeling performance . Greatly saves labor .

4) Alarm Function : The machine will give an alarm when problem occur , such as label spill , label broken , or other malfunctions .

5) Machine Material : The machine and spare parts all use material stainless steel and anodized senior aluminum alloy , with high corrosion resistance and never rust .

6) Equip with a voltage transformer to adapt to local voltage.